VERDA Risk Model Teaching Lab

Software Guide

Software: VERDA Risk Model Lab v1.0

Developer: József Szeles, Department of System Engineering, University of Pannonia

The VERDA Risk Model Lab is a single Windows program with four tabs that holds the fault tree and Bayesian network of the fictional VERDA production cell. Everything recalculates immediately and nothing is saved between sessions, so record settings and results as you go.

1. Installing and starting

| Tab | What you set (left) | What you see |
| --- | --- | --- |
| 1 Fault tree | Basic-event probabilities B1–B10 | Drawn fault tree; cut sets and event details underneath; full text report |
| 2 Sensitivity | Two scale factors | Tornado chart of the top event |
| 3 Bayesian network | Evidence, system changes, all parameters | Drawn network; CPT editor and option comparison underneath |
| 4 Validation | A CSV file of batch records | Observed vs predicted frequencies with a verdict |

- Students: double-click VERDA_Risk_Lab.exe. If Windows SmartScreen warns, choose More info → Run anyway.

- Without the .exe: install Python 3.10+ from python.org (tick Add python.exe to PATH), open a command prompt in the verda folder and run python verda_app.py.

- Instructors building the .exe: install Python, then double-click build_exe.bat in the verda folder; the result is dist\VERDA_Risk_Lab.exe (single file, no Python needed on student PCs).

- The program opens maximised and needs a screen of at least 1280 × 820 pixels.

Back to the original model: Reset defaults (tab 1), Clear evidence and changes and Edit all parameters → Reset defaults (tab 3), or restart the program.

Files in the package: verda_app.py (interface), verda_diagrams.py (diagrams), verda_model.py (model, also usable from Python or Jupyter), generate_data.py (sample data), verda_batch_records.csv (200 sample batches), build_exe.bat, README.md.

2. Tab 1 – Fault tree

Left: basic events. The ten basic events B1–B10 with a short name, layer (Human, Technology, Process, Organisation, Data) and probability per batch.

- To change a probability: type a new value (0–1, decimal point or comma) and press Enter or click Calculate.

- Reset defaults restores all ten values.

- Result shows P(unsafe batch released), the "1 in N batches" equivalent and the change versus the default model.

- Full report (gates, cut sets, importance)… opens a text window with every gate's probability, all minimal cut sets with their share of the top event, and the importance table.

Right: the drawn fault tree. AND/OR gate symbols, each box with its probability. Basic events are shaded by Fussell–Vesely importance: the redder, the more risk they carry.

- Click a basic event in the diagram: its details appear underneath, and its input field on the left is selected so you can type a new value straight away.

- Click a cut set in the list underneath to highlight its events and the path to the top event in blue. Clear highlight removes it.

- Save diagram (.eps) exports a vector image for reports.

Importance measures. Fussell–Vesely = the share of the top-event risk that involves the event ("how much of today's risk does it carry?"). Birnbaum = how much the top event changes if the event goes from never to certain ("how sensitive is the system to it?").

3. Tab 2 – Sensitivity

A tornado chart of the top event when each basic event is scaled down (blue) and up (red), using the values currently set on tab 1.

- Set the two scale factors (default ×0.5 and ×2) and press Enter or click Update.

- Events are sorted by the width of their bar: the top bar moves the top event most.

- Reset defaults sets the factors back to ×0.5 and ×2.

4. Tab 3 – Bayesian network

The Bayesian network adds three root causes that the fault tree treats as independent: handover time pressure (TP), digital instructions down because of Wi-Fi loss (DI) and probe fault (PF).

Left: settings.

- Evidence (what we observe): for nine nodes choose unknown, yes or no. "Yes" means "we know this happened". Other nodes: right-click them in the diagram.

- Changes to the system: tick Second temperature probe, Paper fallback for work instructions or Cyber incident (B10 ×10). Several can be ticked together.

- Clear evidence and changes resets evidence and ticks, not edited parameters.

- Edit all parameters… opens every CPT value in one window (names in section 6.3); Apply to use them, Reset defaults to restore.

- Node probability table… shows all node probabilities now versus without evidence (a snapshot; reopen after changes).

Right: the drawn network. The header shows P(unsafe batch released) and its change versus the same model without evidence, followed by the active changes and evidence. Each box shows the node's probability and change factor.

- Colours: pale red border = observed yes, green border = observed no; red fill = more likely than without evidence, green fill = less likely. Dashed border = root cause. Blue frame = selected node.

- Right-click a node to cycle its evidence: unknown → yes → no. Impossible evidence (e.g. Unsafe release = yes with Contamination = no) is rejected and undone.

- When the model differs from the default, an orange note top right shows how many values changed and the default model's risk.

- Save diagram (.eps) exports the diagram.

Underneath, left: selected node and its CPT. Left-click a node to select it.

- The panel shows the node's type, parents, current probability and evidence buttons.

- Editing a CPT: a probabilistic node lists one row per combination of parent values. Type a new P(yes) and press Enter or Apply; P(no) updates automatically. Reset this node restores its defaults. The grey code at the end of each row (e.g. SS_11) is the parameter name used in Edit all parameters.

- Logic nodes (Added hot, Poor preservation, Nozzle contaminated, Contamination, QC misses it, Unsafe release) have a fixed AND/OR table; to change them, edit their parents.

Underneath, right: Compare improvement options. P(unsafe release) and P(contamination) for each option under the same evidence. Each option is computed alone; ticked changes are not combined here. For a combination, tick the boxes and read the diagram header.

5. Tab 4 – Validation

- Load batch records (CSV)… reads a file with 0/1 columns: shift_handover_pressure, wifi_down, probe_fault, sanitation_skipped, released_before_result, contaminated, unsafe_release.

- For each column the table shows events / batches, the observed frequency, its 95 % confidence interval (Wilson) and the model's prediction with the current tab 3 parameters.

- Verdict: consistent if the model value lies inside the interval, CHECK MODEL (red) if not.

- Generate sample data… writes a fresh set of 200 sample records.

- After changing parameters on tab 3, load the file again to refresh the comparison.

6. Model reference

6.1 Fault tree

Top event: Unsafe batch released = Contamination AND QC fails to detect. Default result: 0.0019 per batch (about 1 in 527 batches).

| Gate | Type | Inputs | Default P |
| --- | --- | --- | --- |
| TOP – Unsafe batch released | AND | CONTAM, QCFAIL | 0.0019 |
| CONTAM – Contamination occurs | OR | PRES, NOZZLE | 0.0257 |
| QCFAIL – QC fails to detect | OR | B8, B9, B10 | 0.0737 |
| PRES – Inadequate preservation | OR | B1, B2, HOT, B5 | 0.0139 |
| HOT – Preservative added above 40 °C | AND | B3, B4 | 0.0030 |
| NOZZLE – Nozzle contamination | OR | B6, B7 | 0.0120 |

| Event | Description | Layer | Default P |
| --- | --- | --- | --- |
| B1 | Preservative under-weighed | Human | 0.005 |
| B2 | Raw material swapped at weighing | Human | 0.002 |
| B3 | Temperature probe reads low | Technology | 0.01 |
| B4 | Operator does not cross-check temperature | Human | 0.3 |
| B5 | pH outside 5.0–5.5 not corrected | Process | 0.004 |
| B6 | Nozzle sanitation skipped at shift handover | Organisation | 0.01 |
| B7 | Sanitiser expired or ineffective | Process | 0.002 |
| B8 | QC sample not representative | Process | 0.05 |
| B9 | Batch released before micro result | Organisation | 0.02 |
| B10 | False "pass" in electronic batch record | Data | 0.005 |

6.2 Bayesian network

The network keeps the fault tree's failure paths and adds three root causes. Default P(unsafe release) = 0.0020.

| Node | Meaning | Parents | Type |
| --- | --- | --- | --- |
| TP | Handover time pressure | – | root cause |
| DI | Digital instructions down (Wi-Fi loss) | – | root cause |
| PF | Temperature probe fault (= B3) | – | root cause |
| B1 | Preservative under-weighed | DI | probabilistic |
| B2 | Raw material swapped | DI | probabilistic |
| B4 | No manual cross-check | – | probabilistic |
| B5 | pH not corrected | – | probabilistic |
| SS | Sanitation skipped (= B6) | TP, DI | probabilistic |
| B7 | Sanitiser ineffective | – | probabilistic |
| RB | Released before result (= B9) | TP | probabilistic |
| B8 | Sample not representative | – | probabilistic |
| B10 | False pass recorded | – | probabilistic |
| HOT | Added above 40 °C | PF, B4 | logic AND |
| PRES | Inadequate preservation | B1, B2, HOT, B5 | logic OR |
| NOZZLE | Nozzle contamination | SS, B7 | logic OR |
| C | Contamination | PRES, NOZZLE | logic OR |
| QM | QC misses it | RB, B8, B10 | logic OR |
| U | Unsafe batch released | C, QM | logic AND |

6.3 CPT parameters

These names appear in Edit all parameters and as grey codes in the CPT editor (both on tab 3). Each value is P(node = yes) for the stated parent condition.

| Parameter | Node | Parent condition | Default |
| --- | --- | --- | --- |
| P_TP | TP | – | 0.2 |
| P_DI_DOWN | DI | – | 0.05 |
| P_PF | PF | – | 0.01 |
| B1_ok / B1_down | B1 | instructions available / down | 0.005 / 0.015 |
| B2_ok / B2_down | B2 | instructions available / down | 0.002 / 0.006 |
| B4 | B4 | – | 0.3 |
| B5 | B5 | – | 0.004 |
| SS_00 | SS | no time pressure, instructions available | 0.005 |
| SS_01 | SS | no time pressure, instructions down | 0.02 |
| SS_10 | SS | time pressure, instructions available | 0.02 |
| SS_11 | SS | time pressure, instructions down | 0.08 |
| B7 | B7 | – | 0.002 |
| RB_0 / RB_1 | RB | no time pressure / time pressure | 0.01 / 0.06 |
| B8 | B8 | – | 0.05 |
| B10 | B10 | – | 0.005 |

What the system changes do: Second temperature probe multiplies P_PF by 0.05. Paper fallback limits the rise when instructions are down to ×1.5 of the "available" value for B1, B2 and SS. Cyber incident multiplies B10 by 10.